RLE0371 Design Spec :

Chapter: Peripheral & MISC Control Register

REVISION HISTORY

| Revision | Release Date | Summary |
| --- | --- | --- |
| 0.1 | 2010/04/07 | Initial for RLE0371 |

1.	Design Note :	3

2.	Register Space Mapping	4

3.	System Control Register	5

3.1.	Interrupt Control	5

3.1.1.	Global Interrupt Mask Register (GIMR) (0xb800-3000)	5

3.1.2.	Global Interrupt Status Register (GISR) (0xB800-3004)	7

3.1.3.	Interrupt Routing Register 0 (IRR0) (0xB800-3008)	8

3.1.4.	Interrupt Routing Register 1 (IRR1) (0xB800-300C)	8

3.1.5.	Interrupt Routing Register 2 (IRR2) (0xB800-3010)	8

3.1.6.	Interrupt Routing Register 3 (IRR3) (0xB800-3014)	9

3.2.	Timer Control	10

3.3.	GPIO Control	14

# Design Note :

| No. | Design Change Description (增修主題/目的/概述) | Reference / Keyword |
| --- | --- | --- |

# Register Space Mapping

Table 1.   MISC Control Space

| Name | Mode | Cached? | Virtual Address | Physical Address | Size | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Switch Core Control |  |  |  |  |  |  |
|  |  |  | 0xB800_3000 | 0x1800_3000 | 4KB | MISC ( timer, arbiter, interrupt controller, GPIO, clock …) |
|  |  |  | 0xB800_3FFF | 0x1800_3FFF |  |  |
| KSEG1 | Kernel | Uncached | 0xB800_3000 -0xB800_30FF | 1 | 64W | Interrupt Control 0 Register |
|  |  |  | 0xB800_3100 -0xB800_31FF | 2 | 64W | Interrupt Control 1 Register |
|  |  |  | 0xB800_3200 -0xB800_32FF | 3 | 64W | Timer |
|  |  |  | 0xB800_3300 -0xB800_33FF | 4 | 64W | GPIO Control Register |
|  |  |  | 0xB800_3400 -0xB800_34FF | 5 | 64W |  |
|  |  |  | 0xB800_3500 -0xB800_35FF | 6 | 64W |  |
|  |  |  | 0xB800_3600 -0xB800_36FF | 7 | 64W |  |
|  |  |  | 0xB800_3700 -0xB800_37FF | 8 | 64W |  |
|  |  |  | 0xB800_3800 -0xB800_38FF | 9 | 64W |  |
|  |  |  | 0xB800_3900 -0xB800_39FF | 10 | 64W |  |
|  |  |  | 0xB800_3A00 -0xB800_3AFF | 11 | 64W |  |
|  |  |  | 0xB800_3B00 -0xB800_3BFF | 12 | 64W |  |
|  |  |  | 0xB800_3C00 -0xB800_3CFF | 13 | 64W |  |
|  |  |  | 0xB800_3D00 -0xB800_3DFF | 14 | 64W |  |
|  |  |  | 0xB800_3E00 -0xB800_3EFF | 15 | 64W |  |
|  |  |  | 0xB800_3F00 -0xB800_3FFF | 16 | 64W |  |

註明 :

分類別:

# 3.	System Control Register

## 3.1.	Interrupt Control

The RLE0371 (LX5281) provides 14 hardware-interrupt inputs, IRQ2 to IRQ15. The Global Interrupt Mask Register (GIMR) enables or disables an interrupt coming from the Timer, USB, UART, PCIE, GPIO, or Timer modules. The Global Interrupt Status Register (GISR) shows the pending interrupt status. The Interrupt Routing Register (IRR) controls the mapping from the interrupt source to IRQ 2~15.

Table 1.  Interrupt Control_0 Register Address Mapping (Base = 0xB800-3000)

| Offset | Size (byte) | Name | Description | Field |
| --- | --- | --- | --- | --- |
| 00 | 4 | GIMR0_0 | Global interrupt mask 0 register for CPU0 |  |
| 04 | 4 | GIMR1_0 | Global interrupt mask 1 register for CPU0. |  |
| 08 | 4 | GISR0_0 | Global interrupt status 0 register for CPU0. |  |
| 0C | 4 | GISR1_0 | Global interrupt status 1 register for CPU0. |  |
| 10 | 4 | IRR0_0 | Interrupt routing register 0 for CPU0 |  |
| 14 | 4 | IRR1_0 | Interrupt routing register 1 for CPU0 |  |
| 18 | 4 | IRR2_0 | Interrupt routing register 2 for CPU0 |  |
| 1C | 4 | IRR3_0 | Interrupt routing register 3 for CPU0 |  |
| 20 | 4 | IRR4_0 | Interrupt routing register 4 for CPU0 |  |
| 24 | 4 | IRR5_0 | Interrupt routing register 5 for CPU0 |  |

Table 2.  Global Interrupt Mask Register (GIMR0_0)   (0xB800_3000)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31 | TMO_IE | time-out interrupt enable | R/W | 0 |
| 30 | VOIPACC_IE | VOIPACC interrupt enable. | R/W | 0 |
| 29 | SPI_IE | SPI (for VoIP SLIC) interrupt enable | R/W | 0 |
| 28 | PTM_IE | PTM interrupt enable | R/W | 0 |
| 27 | PKT_IE | Packet Processor (Accelerator) interrupt enable | R/W | 0 |
| 26 | GMAC_IE | GMAC interrupt enable | R/W | 0 |
| 25 | SAR_IE | SAR interrupt enable | R/W | 0 |
| 24 | PKT_GMAC_IE | From packet processor for GMAC interrupt enable. | R/W | 0 |
| 23 | PKT_XTM_IE | From packet processor for SAR/PTM interrupt enable. | R/W | 0 |
| 22 | DMT_IE | DMT interrupt enable | R/W | 0 |
| 21 | GDMA1_IE | Generic DMA 1 interrupt enable | R/W | 0 |
| 20 | GDMA0_IE | Generic DMA 0 interrupt enable | R/W | 0 |
| 19 | SECURITY_IE | Security Engine interrupt enable | R/W | 0 |
| 18 | PCM1_IE | PCM1 interface interrupt enable | R/W | 0 |
| 17 | PCM0_IE | PCM0 interface interrupt enable | R/W | 0 |
| 16 | PCIE1_IE | PCIE_1 interrupt enable. | R/W | 0 |
| 15 | PCIE0_IE | PCIE_0 interrupt enable. | R/W | 0 |
| 14 | USB_H2_IE | USB 2.0 Host interrupt enable. | R/W | 0 |
| 13 | USB_H3_IE | USB 3.0 Host interrupt enable. | R/W | 0 |
| 12 | PERIPHERAL_IE | Peripheral interrupt enable. | R/W | 0 |
| 11 | RSVD | CMU 1 interrupt enable | R/W | 0 |
| 10 | RSVD | CMU 0 interrupt enable | R/W | 0 |
| 9 | RSVD | Speed Detect interrupt enable | R/W | 0 |
| 8 | SWITCH_IE | SWITCH interrupt enable | R/W | 0 |
| 7 | SATA_IE | SATA interrupt enable | R/W | 0 |
| 6 | NFBI_IE | NFBI interrupt enable | R/W | 0 |
| 5 | WDOG_IE | WDOG interrupt enable | R/W | 0 |
| 4 | FFTACC_IE | FFTACC interrupt enable | R/W | 0 |
| 3 | FLSH_IE | NAND FLASH interrupt enable | R/W | 0 |
| 2 | UTMD_IE | UTOPIA MDIO interrupt enable | R/W | 0 |
| 1-0 | RSVD |  |  |  |

Table 3.  Global Interrupt Mask Register (GIMR1_0)   (0xB800_3004)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31 | LBCTMOm2_IE | LBC time-out master 2 interrupt enable | R/W | 0 |
| 30 | LBCTMOm1_IE | LBC time-out master 1 interrupt enable | R/W | 0 |
| 29 | LBCTMOm0_IE | LBC time-out master 0 interrupt enable | R/W | 0 |
| 28 | LBCTMOs3_IE | LBC time-out slave 2 interrupt enable | R/W | 0 |
| 27 | LBCTMOs2_IE | LBC time-out slave 2 interrupt enable | R/W | 0 |
| 26 | LBCTMOs1_IE | LBC time-out slave 1 interrupt enable | R/W | 0 |
| 25 | LBCTMOs0_IE | LBC time-out slave 0 interrupt enable | R/W | 0 |
| 24 | OCPTO1_IE | OCP Bus time-out 1 interrupt enable. | R/W | 0 |
| 23 | OCPTO0_IE | OCP Bus time-out 0 interrupt enable. | R/W | 0 |
| 22 | OCP1_CPU0_ILA_IP | OCP1_CPU illegal address interrupt pending flag. (level trigger type) | R/W | 0 |
| 21 | OCP0_CPU1_ILA_IP | OCP0_CPU illegal address interrupt pending flag. (level trigger type) | R/W | 0 |
| 20 | UART3_IE | UART3 interrupt enable | R/W | 0 |
| 19 | UART2_IE | UART2 interrupt enable | R/W | 0 |
| 18 | UART1_IE | UART1 interrupt enable | R/W | 0 |
| 17 | UART0_IE | UART0 interrupt enable | R/W | 0 |
| 16 | TC5_IE | Timers/Counters #5 interrupt enable. | R/W | 0 |
| 15 | TC4_IE | Timers/Counters #4 interrupt enable. | R/W | 0 |
| 14 | TC3_IE | Timers/Counters #3 interrupt enable. | R/W | 0 |
| 13 | TC2_IE | Timers/Counters #2 interrupt enable. | R/W | 0 |
| 12 | TC1_IE | Timers/Counters #1 interrupt enable. | R/W | 0 |
| 11 | TC0_IE | Timers/Counters #0 interrupt enable. | R/W | 0 |
| 10 | GPIO_IE | GPIO Port A,B,C,D interrupt enable | R/W | 0 |
| 9 | GPIO_IE | GPIO Port E,F,G,H interrupt enable | R/W | 0 |
| 8-0 | RSVD |  |  |  |

Table 4.  Global Interrupt Status Register (GISR0_0)   (0xB800_3008)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31 | TMO_IP | time-out interrupt pending flag. | R/W | 0 |
| 30 | VOIPACC_IP | VOIPACC interrupt pending flag. (level trigger type) | R/W | 0 |
| 29 | SPI_IP | SPI (for VoIP SLIC, not for SPI Flash) interrupt pending flag. (level trigger type) | R/W | 0 |
| 28 | PTM_IP | PTM interrupt pending flag. (level trigger type) | R/W | 0 |
| 27 | PKT_IP | Packet Processor (Accelerator) interrupt pending flag. (level trigger type) | R/W | 0 |
| 26 | GMAC_IP | GMAC interrupt pending flag. (level trigger type) | R/W | 0 |
| 25 | SAR_IP | SAR interrupt pending flag. (level trigger type) | R/W | 0 |
| 24 | PKT_GMAC_IP | From packet processor for GMAC interrupt pending flag.. (level trigger type). | R/W | 0 |
| 23 | PKT_DMT_IP | From packet processor for SAR/PTM interrupt pending flag.. | R/W | 0 |
| 22 | DMT_IP | DMT interrupt pending flag. | R/W | 0 |
| 21 | GDMA1_IP | Generic DMA 1 interrupt pending flag. | R/W | 0 |
| 20 | GDMA0_IP | Generic DMA 0 interrupt pending flag. | R/W | 0 |
| 19 | SECURITY_IP | Security Engine interrupt pending flag. | R/W | 0 |
| 18 | PCM0_IP | PCM interface interrupt enable for cpu0 | R/W | 0 |
| 17 | PCM1_IP | PCM interface interrupt enable for cpu1 | R/W | 0 |
| 16 | PCIE1_IP | PCIE_1 interrupt pending flag.. | R/W | 0 |
| 15 | PCIE0_IP | PCIE_0 interrupt pending flag.. | R/W | 0 |
| 14 | USB_H2_IP | USB 2.0 Host interrupt pending flag.. | R/W | 0 |
| 13 | USB_H3_IP | USB 3.0 Host interrupt pending flag.. | R/W | 0 |
| 12 | PERIPHERAL_IP | Peripheral interrupt pending flag.. | R/W | 0 |
| 11 | RSVD | CMU 1 interrupt pending flag. | R/W | 0 |
| 10 | RSVD | CMU 0 interrupt pending flag. | R/W | 0 |
| 9 | RSVD | Speed Detect interrupt pending flag. | R/W | 0 |
| 8 | SWITCH_IP | SWITCH interrupt pending flag. | R/W | 0 |
| 7 | SATA_IP | SATA interrupt pending flag. | R/W | 0 |
| 6 | NFBI_IP | NFBI interrupt pending flag. | R/W | 0 |
| 5 | WDOG1_IP | WATCH DOG 0 interrupt pending flag. | R/W | 0 |
| 4 | FFTACC_IP | FFTACC interrupt pending flag. | R/W | 0 |
| 3 | FLSH_IP | NAND FLASH interrupt pending flag | R/W | 0 |
| 2 | UTMD_IP | UTOPIA MDIO interrupt pending flag | R/W | 0 |
| 1-0 | RSVD |  |  |  |

Table 5.  Global Interrupt Status Register (GISR1__0)   (0xB800_300C)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31 | LBCTMOm2_IP | LBC time-out master 2 interrupt pending flag (pulse trigger type) | R/W | 0 |
| 30 | LBCTMOm1_IP | LBC time-out master 1 interrupt pending flag (pulse trigger type) | R/W | 0 |
| 29 | LBCTMOm0_IP | LBC time-out master 0 interrupt pending flag (pulse trigger type) | R/W | 0 |
| 28 | LBCTMOs3_IP | LBC time-out slave 3 interrupt pending flag (pulse trigger type) | R/W | 0 |
| 27 | LBCTMOs2_IP | LBC time-out slave 2 interrupt pending flag (pulse trigger type) | R/W | 0 |
| 26 | LBCTMOs1_IP | LBC time-out slave 1 interrupt pending flag (pulse trigger type) | R/W | 0 |
| 25 | LBCTMOs0_IP | LBC time-out slave 0 interrupt pending flag (pulse trigger type) | R/W | 0 |
| 24 | OCPTO1_IP | OCP Bus time-out 1 interrupt pending flag. (level trigger type) | R/W | 0 |
| 23 | OCPTO0_IP | OCP Bus time-out 0 interrupt pending flag. (level trigger type) | R/W | 0 |
| 22 | OCP1_CPU0_ILA_IP | OCP1_CPU illegal address interrupt pending flag. (level trigger type) | R/W | 0 |
| 21 | OCP0_CPU1_ILA_IP | OCP0_CPU illegal address interrupt pending flag. (level trigger type) | R/W | 0 |
| 20 | UART3_IP | UART3 interrupt pending flag. (level trigger type) | R/W | 0 |
| 19 | UART2_IP | UART2 interrupt pending flag. (level trigger type) | R/W | 0 |
| 18 | UART1_IP | UART1 interrupt pending flag. (level trigger type) | R/W | 0 |
| 17 | UART0_IP | UART0 interrupt pending flag. (level trigger type) | R/W | 0 |
| 16 | TC5_IP | Timers/Counters #5 interrupt pending flag. (level trigger type) | R/W | 0 |
| 15 | TC4_IP | Timers/Counters #4 interrupt pending flag. (level trigger type) | R/W | 0 |
| 14 | TC3_IP | Timers/Counters #3 interrupt pending flag. (level trigger type) | R/W | 0 |
| 13 | TC2_IP | Timers/Counters #2 interrupt pending flag. (level trigger type) | R/W | 0 |
| 12 | TC1_IP | Timers/Counters #1 interrupt pending flag. (level trigger type) | R/W | 0 |
| 11 | TC0_IP | Timers/Counters #0 interrupt pending flag. (level trigger type) | R/W | 0 |
| 10 | GPIO_IP | GPIO Port A,B,C,D interrupt pending flag. (level trigger type) | R/W | 0 |
| 9 | GPIO_IP | GPIO Port E,F,G,H interrupt pending flag (level trigger type) | R/W | 0 |
| 8-0 | RSVD |  |  |  |

Table 2.  Interrupt Routing Register 0 (IRR0_0)   (0xB800_3010)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31-n.28 | LBCTMOm2_ RS[3:0] | LBC time-out master 2. | R/W | 0 |
| n.27-n.24 | LBCTMOm1_ RS[3:0] | LBC time-out master 1. | R/W | 0 |
| n.23-n.20 | LBCTMOm0_RS[3:0] | LBC time-out master 0. | R/W | 0 |
| n.19-n.16 | LBCTMOs3_RS[3:0] | LBC time-out slave 2. | R/W | 0 |
| n.15-n.12 | LBCTMOs2_RS[3:0] | LBC time-out slave 2. | R/W | 0 |
| n.11-n.8 | LBCTMOs1_RS[3:0] | LBC time-out slave 1. | R/W | 0 |
| n.7-n.4 | LBCTMOs0_RS[3:0] | LBC time-out slave 0. | R/W | 0 |
| n.3-n.0 | OCPTO1_RS[3:0] | OCP Bus time-out 1. | R/W | 0 |

Table 3.  Interrupt Routing Register 1 (IRR1_0)   (0xB800_3014)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31-n.28 | OCPTO0_RS[3:0] | OCP Bus time-out 0. | R/W | 0 |
| n.27-n.24 | OCP1_CPU0_ILA_RS[3:0] | OCP1_CPU illegal address interrupt route select. | R/W | 0 |
| n.23-n.20 | OCP0_CPU1_ILA_RS[3:0] | OCP0_CPU illegal address interrupt route select. | R/W | 0 |
| n.19-n.16 | VOIPACC_RS | VOIPACC interrupt route select. | R/W | 0 |
| n.15-n.12 | SPI_RS[3:0] | VOIP_SPI interrupt route select. | R/W | 0 |
| n.11-n.8 | PTM _RS[3:0] | PTM interrupt route select. | R/W | 0 |
| n.7-n.4 | PKT_RS[3:0] | PKT interrupt route select. | R/W | 0 |
| n.3-n.0 | GMAC_RS[3:0] | GMAC interrupt route select. | R/W | 0 |

Table 4.  Interrupt Routing Register 2 (IRR2_0)   (0xB800_3018)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31-n.28 | SAR _RS[3:0] | SAR interrupt route select. | R/W | 0 |
| n.27-n.24 | PKT_GMAC_RS[3:0] | From packet processor for GMAC interrupt route select. | R/W | 0 |
| n.23-n.20 | PKT_XTM_RS[3:0] | From packet processor for SAR/PTM interrupt route select. | R/W | 0 |
| n.19-n.16 | DMT_RS[3:0] | DMT engine interrupt route select. | R/W | 0 |
| n.15-n.12 | GDMA1_RS[3:0] | Generic DMA 1 interrupt route select | R/W | 0 |
| n.11-n.8 | GDMA0_RS[3:0] | Generic DMA 1 interrupt route select | R/W | 0 |
| n.7-n.4 | Security_RS[3:0] | Security Engine interrupt route select | R/W | 0 |
| n.3-n.0 | PCM1 _RS[3:0] | PCM1 interrupt route select. | R/W | 0 |

Table 5.  Interrupt Routing Register 3 (IRR3_0)   (0xB800_301C)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31-n.28 | PCM0_RS[3:0] | PCM0 interrupt route select. | R/W | 0 |
| n.27-n.24 | PCIE1_RS[3:0] | PCIE1 interrupt route select | R/W | 0 |
| n.23-n.20 | PCIE0_RS[3:0] | PCIE0 interrupt route select. | R/W | 0 |
| n.19-n.16 | USB_H2_RS[3:0] | USB_Host2.0 interrupt route select. | R/W | 0 |
| n.15-n.12 | USB_H3_RS[3:0] | USB_Host3.0 interrupt route select. | R/W | 0 |
| n.11-n.8 | UART3_RS[3:0] | UART3 interrupt route select. | R/W | 0 |
| n.7-n.4 | UART2_RS[3:0] | UART2 interrupt route select. | R/W | 0 |
| n.3-n.0 | UART1_RS[3:0] | UART1 interrupt route select. | R/W | 0 |

Table 10.  Interrupt Routing Register 4 (IRR4_0)   (0xB800_3020)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31-n.28 | UART0_RS[3:0] | UART0 interrupt route select. | R/W | 0 |
| n.27-n.24 | TC5_RS[3:0] | Timers/Counters #5 interrupt route select | R/W | 0 |
| n.23-n.20 | TC4_RS[3:0] | Timers/Counters #4 interrupt route select | R/W | 0 |
| n.19-n.16 | TC3_RS[3:0] | Timers/Counters #3 interrupt route select | R/W | 0 |
| n.15-n.12 | TC2_RS[3:0] | Timers/Counters #2 interrupt route select | R/W | 0 |
| n.11-n.8 | TC1_RS[3:0] | Timers/Counters #1 interrupt route select | R/W | 0 |
| n.7-n.4 | TC0_RS[3:0] | Timers/Counters #0 interrupt route select | R/W | 0 |
| n.3-n.0 | GPIO 1_RS[3:0] | GPIO Port A,B,C,D interrupt route select. | R/W | 0 |

Table 11.  Interrupt Routing Register 5 (IRR5_0)   (0xB800_3024)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31-n.28 | GPIO 0_RS[3:0] | GPIO Port E,F,G,H interrupt route select. | R/W | 0 |
| n.27-n.24 | SWITCH_RS[3:0] | SWITCH interrupt route select | R/W | 0 |
| n.23-n.20 | SATA_RS[3:0] | SATA interrupt route select | R/W | 0 |
| n.19-n.16 | NFBI_RS[3:0] | NFBI interrupt route select | R/W | 0 |
| n.15-n.12 | WDOG_RS[3:0] | WDOG interrupt route select | R/W | 0 |
| n.11-n.8 | FFTACC_RS[3:0] | FFTACC interrupt route select | R/W | 0 |
| n.7-n.4 | FLSH_RS[3:0] | NAND FLSH interrupt route select | R/W | 0 |
| n.3-n.0 | UTMD_RS[3:0] | UTOPIA MDIO interrupt route select | R/W | 0 |

Table 12.  Interrupt Control_1 Register Address Mapping (Base = 0xB800-3100)

| Offset | Size (byte) | Name | Description | Field |
| --- | --- | --- | --- | --- |
| 00 | 4 | GIMR0 | Global interrupt mask 0 register for CPU1. |  |
| 04 | 4 | GIMR1 | Global interrupt mask 1 register for CPU1. |  |
| 08 | 4 | GISR0 | Global interrupt status 0 register for CPU1. |  |
| 0C | 4 | GISR1 | Global interrupt status 1 register for CPU1. |  |
| 10 | 4 | IRR0 | Interrupt routing register 0 for CPU1 |  |
| 14 | 4 | IRR1 | Interrupt routing register 1 for CPU1 |  |
| 18 | 4 | IRR2 | Interrupt routing register 2 for CPU1 |  |
| 1C | 4 | IRR3 | Interrupt routing register 3 for CPU1 |  |
| 20 | 4 | IRR4 | Interrupt routing register 4 for CPU1 |  |
| 24 | 4 | IRR5 | Interrupt routing register 5 for CPU1 |  |

Table 13.  Global Interrupt Mask Register (GIMR0_1)   (0xB800_3100)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31 | TMO_IE | time-out interrupt enable | R/W | 0 |
| 30 | VOIPACC_IE | VOIPACC interrupt enable. | R/W | 0 |
| 29 | SPI_IE | SPI (for VoIP SLIC) interrupt enable | R/W | 0 |
| 28 | PTM_IE | PTM interrupt enable | R/W | 0 |
| 27 | PKT_IE | Packet Processor (Accelerator) interrupt enable | R/W | 0 |
| 26 | GMAC_IE | GMAC interrupt enable | R/W | 0 |
| 25 | SAR_IE | SAR interrupt enable | R/W | 0 |
| 24 | PKT_GMAC_IE | From packet processor for GMAC interrupt enable. | R/W | 0 |
| 23 | PKT_XTM_IE | From packet processor for SAR/PTM interrupt enable. | R/W | 0 |
| 22 | DMT_IE | DMT interrupt enable | R/W | 0 |
| 21 | GDMA1_IE | Generic DMA 1 interrupt enable | R/W | 0 |
| 20 | GDMA0_IE | Generic DMA 0 interrupt enable | R/W | 0 |
| 19 | SECURITY_IE | Security Engine interrupt enable | R/W | 0 |
| 18 | PCM1_IE | PCM1 interface interrupt enable | R/W | 0 |
| 17 | PCM0_IE | PCM0 interface interrupt enable | R/W | 0 |
| 16 | PCIE1_IE | PCIE_1 interrupt enable. | R/W | 0 |
| 15 | PCIE0_IE | PCIE_0 interrupt enable. | R/W | 0 |
| 14 | USB_H2_IE | USB 2.0 Host interrupt enable. | R/W | 0 |
| 13 | USB_H3_IE | USB 3.0 Host interrupt enable. | R/W | 0 |
| 12 | PERIPHERAL_IE | Peripheral interrupt enable. | R/W | 0 |
| 11 | RSVD | CMU 1 interrupt enable | R/W | 0 |
| 10 | RSVD | CMU 0 interrupt enable | R/W | 0 |
| 9 | RSVD | Speed Detect interrupt enable | R/W | 0 |
| 8 | SWITCH_IE | SWITCH interrupt enable | R/W | 0 |
| 7 | SATA_IE | SATA interrupt enable | R/W | 0 |
| 6 | NFBI_IE | NFBI interrupt enable | R/W | 0 |
| 5 | WDOG_IE | WDOG interrupt enable | R/W | 0 |
| 4 | FFTACC_IE | FFTACC interrupt enable | R/W | 0 |
| 3 | FLSH_IE | NAND FLASH interrupt enable | R/W | 0 |
| 2 | UTMD_IE | UTOPIA MDIO interrupt enable | R/W | 0 |
| 1-0 | RSVD |  |  |  |

Table 14.  Global Interrupt Mask Register (GIMR1_1)   (0xB800_3104)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31 | LBCTMOm2_IE | LBC time-out master 2 interrupt enable | R/W | 0 |
| 30 | LBCTMOm1_IE | LBC time-out master 1 interrupt enable | R/W | 0 |
| 29 | LBCTMOm0_IE | LBC time-out master 0 interrupt enable | R/W | 0 |
| 28 | LBCTMOs3_IE | LBC time-out slave 2 interrupt enable | R/W | 0 |
| 27 | LBCTMOs2_IE | LBC time-out slave 2 interrupt enable | R/W | 0 |
| 26 | LBCTMOs1_IE | LBC time-out slave 1 interrupt enable | R/W | 0 |
| 25 | LBCTMOs0_IE | LBC time-out slave 0 interrupt enable | R/W | 0 |
| 24 | OCPTO1_IE | OCP Bus time-out 1 interrupt enable. | R/W | 0 |
| 23 | OCPTO0_IE | OCP Bus time-out 0 interrupt enable. | R/W | 0 |
| 22 | OCP1_CPU0_ILA_IP | OCP1_CPU illegal address interrupt pending flag. (level trigger type) | R/W | 0 |
| 21 | OCP0_CPU1_ILA_IP | OCP0_CPU illegal address interrupt pending flag. (level trigger type) | R/W | 0 |
| 20 | UART3_IE | UART3 interrupt enable | R/W | 0 |
| 19 | UART2_IE | UART2 interrupt enable | R/W | 0 |
| 18 | UART1_IE | UART1 interrupt enable | R/W | 0 |
| 17 | UART0_IE | UART0 interrupt enable | R/W | 0 |
| 16 | TC5_IE | Timers/Counters #5 interrupt enable. | R/W | 0 |
| 15 | TC4_IE | Timers/Counters #4 interrupt enable. | R/W | 0 |
| 14 | TC3_IE | Timers/Counters #3 interrupt enable. | R/W | 0 |
| 13 | TC2_IE | Timers/Counters #2 interrupt enable. | R/W | 0 |
| 12 | TC1_IE | Timers/Counters #1 interrupt enable. | R/W | 0 |
| 11 | TC0_IE | Timers/Counters #0 interrupt enable. | R/W | 0 |
| 10 | GPIO_IE | GPIO Port A,B,C,D interrupt enable | R/W | 0 |
| 9 | GPIO_IE | GPIO Port E,F,G,H interrupt enable | R/W | 0 |
| 8-0 | RSVD |  |  |  |

Table 15.  Global Interrupt Status Register (GISR0_1)   (0xB800_3108)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31 | TMO_IP | time-out interrupt pending flag. | R/W | 0 |
| 30 | VOIPACC_IP | VOIPACC interrupt pending flag. (level trigger type) | R/W | 0 |
| 29 | SPI_IP | SPI (for VoIP SLIC, not for SPI Flash) interrupt pending flag. (level trigger type) | R/W | 0 |
| 28 | PTM_IP | PTM interrupt pending flag. (level trigger type) | R/W | 0 |
| 27 | PKT_IP | Packet Processor (Accelerator) interrupt pending flag. (level trigger type) | R/W | 0 |
| 26 | GMAC_IP | GMAC interrupt pending flag. (level trigger type) | R/W | 0 |
| 25 | SAR_IP | SAR interrupt pending flag. (level trigger type) | R/W | 0 |
| 24 | PKT_GMAC_IP | From packet processor for GMAC interrupt pending flag.. (level trigger type). | R/W | 0 |
| 23 | PKT_DMT_IP | From packet processor for SAR/PTM interrupt pending flag.. | R/W | 0 |
| 22 | DMT_IP | DMT interrupt pending flag. | R/W | 0 |
| 21 | GDMA1_IP | Generic DMA 1 interrupt pending flag. | R/W | 0 |
| 20 | GDMA0_IP | Generic DMA 0 interrupt pending flag. | R/W | 0 |
| 19 | SECURITY_IP | Security Engine interrupt pending flag. | R/W | 0 |
| 18 | PCM0_IP | PCM interface interrupt enable for cpu0 | R/W | 0 |
| 17 | PCM1_IP | PCM interface interrupt enable for cpu1 | R/W | 0 |
| 16 | PCIE1_IP | PCIE_1 interrupt pending flag.. | R/W | 0 |
| 15 | PCIE0_IP | PCIE_0 interrupt pending flag.. | R/W | 0 |
| 14 | USB_H2_IP | USB 2.0 Host interrupt pending flag.. | R/W | 0 |
| 13 | USB_H3_IP | USB 3.0 Host interrupt pending flag.. | R/W | 0 |
| 12 | PERIPHERAL_IP | Peripheral interrupt pending flag.. | R/W | 0 |
| 11 | RSVD | CMU 1 interrupt pending flag. | R/W | 0 |
| 10 | RSVD | CMU 0 interrupt pending flag. | R/W | 0 |
| 9 | RSVD | Speed Detect interrupt pending flag. | R/W | 0 |
| 8 | SWITCH_IP | SWITCH interrupt pending flag. | R/W | 0 |
| 7 | SATA_IP | SATA interrupt pending flag. | R/W | 0 |
| 6 | NFBI_IP | NFBI interrupt pending flag. | R/W | 0 |
| 5 | WDOG1_IP | WATCH DOG 0 interrupt pending flag. | R/W | 0 |
| 4 | FFTACC_IP | FFTACC interrupt pending flag. | R/W | 0 |
| 3 | FLSH_IP | NAND FLASH interrupt pending flag | R/W | 0 |
| 2 | UTMD_IP | UTOPIA MDIO interrupt pending flag | R/W | 0 |
| 1-0 | RSVD |  |  |  |

Table 16.  Global Interrupt Status Register (GISR1_1)   (0xB800_310C)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31 | LBCTMOm2_IP | LBC time-out master 2 interrupt pending flag (pulse trigger type) | R/W | 0 |
| 30 | LBCTMOm1_IP | LBC time-out master 1 interrupt pending flag (pulse trigger type) | R/W | 0 |
| 29 | LBCTMOm0_IP | LBC time-out master 0 interrupt pending flag (pulse trigger type) | R/W | 0 |
| 28 | LBCTMOs3_IP | LBC time-out slave 3 interrupt pending flag (pulse trigger type) | R/W | 0 |
| 27 | LBCTMOs2_IP | LBC time-out slave 2 interrupt pending flag (pulse trigger type) | R/W | 0 |
| 26 | LBCTMOs1_IP | LBC time-out slave 1 interrupt pending flag (pulse trigger type) | R/W | 0 |
| 25 | LBCTMOs0_IP | LBC time-out slave 0 interrupt pending flag (pulse trigger type) | R/W | 0 |
| 24 | OCPTO1_IP | OCP Bus time-out 1 interrupt pending flag. (level trigger type) | R/W | 0 |
| 23 | OCPTO0_IP | OCP Bus time-out 0 interrupt pending flag. (level trigger type) | R/W | 0 |
| 22 | OCP1_CPU0_ILA_IP | OCP1_CPU illegal address interrupt pending flag. (level trigger type) | R/W | 0 |
| 21 | OCP0_CPU1_ILA_IP | OCP0_CPU illegal address interrupt pending flag. (level trigger type) | R/W | 0 |
| 20 | UART3_IP | UART3 interrupt pending flag. (level trigger type) | R/W | 0 |
| 19 | UART2_IP | UART2 interrupt pending flag. (level trigger type) | R/W | 0 |
| 18 | UART1_IP | UART1 interrupt pending flag. (level trigger type) | R/W | 0 |
| 17 | UART0_IP | UART0 interrupt pending flag. (level trigger type) | R/W | 0 |
| 16 | TC5_IP | Timers/Counters #5 interrupt pending flag. (level trigger type) | R/W | 0 |
| 15 | TC4_IP | Timers/Counters #4 interrupt pending flag. (level trigger type) | R/W | 0 |
| 14 | TC3_IP | Timers/Counters #3 interrupt pending flag. (level trigger type) | R/W | 0 |
| 13 | TC2_IP | Timers/Counters #2 interrupt pending flag. (level trigger type) | R/W | 0 |
| 12 | TC1_IP | Timers/Counters #1 interrupt pending flag. (level trigger type) | R/W | 0 |
| 11 | TC0_IP | Timers/Counters #0 interrupt pending flag. (level trigger type) | R/W | 0 |
| 10 | GPIO_IP | GPIO Port A,B,C,D interrupt pending flag. (level trigger type) | R/W | 0 |
| 9 | GPIO_IP | GPIO Port E,F,G,H interrupt pending flag (level trigger type) | R/W | 0 |
| 8-0 | RSVD |  |  |  |

Table 17.  Interrupt Routing Register 0 (IRR0_1)   (0xB800_3110)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31-n.28 | LBCTMOm2_ RS[3:0] | LBC time-out master 2. | R/W | 0 |
| n.27-n.24 | LBCTMOm1_ RS[3:0] | LBC time-out master 1. | R/W | 0 |
| n.23-n.20 | LBCTMOm0_RS[3:0] | LBC time-out master 0. | R/W | 0 |
| n.19-n.16 | LBCTMOs3_RS[3:0] | LBC time-out slave 2. | R/W | 0 |
| n.15-n.12 | LBCTMOs2_RS[3:0] | LBC time-out slave 2. | R/W | 0 |
| n.11-n.8 | LBCTMOs1_RS[3:0] | LBC time-out slave 1. | R/W | 0 |
| n.7-n.4 | LBCTMOs0_RS[3:0] | LBC time-out slave 0. | R/W | 0 |
| n.3-n.0 | OCPTO1_RS[3:0] | OCP Bus time-out 1. | R/W | 0 |

Table 18.  Interrupt Routing Register 1 (IRR1_1)   (0xB800_3114)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31-n.28 | OCPTO0_RS[3:0] | OCP Bus time-out 0. | R/W | 0 |
| n.27-n.24 | OCP1_CPU0_ILA_RS[3:0] | OCP1_CPU illegal address interrupt route select. | R/W | 0 |
| n.23-n.20 | OCP0_CPU1_ILA_RS[3:0] | OCP0_CPU illegal address interrupt route select. | R/W | 0 |
| n.19-n.16 | VOIPACC_RS | VOIPACC interrupt route select. | R/W | 0 |
| n.15-n.12 | SPI_RS[3:0] | VOIP_SPI interrupt route select. | R/W | 0 |
| n.11-n.8 | PTM _RS[3:0] | PTM interrupt route select. | R/W | 0 |
| n.7-n.4 | PKT_RS[3:0] | PKT interrupt route select. | R/W | 0 |
| n.3-n.0 | GMAC_RS[3:0] | GMAC interrupt route select. | R/W | 0 |

Table 19.  Interrupt Routing Register 2 (IRR2_1)   (0xB800_3118)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31-n.28 | SAR _RS[3:0] | SAR interrupt route select. | R/W | 0 |
| n.27-n.24 | PKT_GMAC_RS[3:0] | From packet processor for GMAC interrupt route select. | R/W | 0 |
| n.23-n.20 | PKT_XTM_RS[3:0] | From packet processor for SAR/PTM interrupt route select. | R/W | 0 |
| n.19-n.16 | DMT_RS[3:0] | DMT engine interrupt route select. | R/W | 0 |
| n.15-n.12 | GDMA1_RS[3:0] | Generic DMA 1 interrupt route select | R/W | 0 |
| n.11-n.8 | GDMA0_RS[3:0] | Generic DMA 1 interrupt route select | R/W | 0 |
| n.7-n.4 | Security_RS[3:0] | Security Engine interrupt route select | R/W | 0 |
| n.3-n.0 | PCM1 _RS[3:0] | PCM1 interrupt route select. | R/W | 0 |

Table 20.  Interrupt Routing Register 3 (IRR3_1)   (0xB800_311C)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31-n.28 | PCM0_RS[3:0] | PCM0 interrupt route select. | R/W | 0 |
| n.27-n.24 | PCIE1_RS[3:0] | PCIE1 interrupt route select | R/W | 0 |
| n.23-n.20 | PCIE0_RS[3:0] | PCIE0 interrupt route select. | R/W | 0 |
| n.19-n.16 | USB_H2_RS[3:0] | USB_Host2.0 interrupt route select. | R/W | 0 |
| n.15-n.12 | USB_H3_RS[3:0] | USB_Host3.0 interrupt route select. | R/W | 0 |
| n.11-n.8 | UART3_RS[3:0] | UART3 interrupt route select. | R/W | 0 |
| n.7-n.4 | UART2_RS[3:0] | UART2 interrupt route select. | R/W | 0 |
| n.3-n.0 | UART1_RS[3:0] | UART1 interrupt route select. | R/W | 0 |

Table 21.  Interrupt Routing Register 4 (IRR4_1)   (0xB800_3120)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31-n.28 | UART0_RS[3:0] | UART0 interrupt route select. | R/W | 0 |
| n.27-n.24 | TC5_RS[3:0] | Timers/Counters #5 interrupt route select | R/W | 0 |
| n.23-n.20 | TC4_RS[3:0] | Timers/Counters #4 interrupt route select | R/W | 0 |
| n.19-n.16 | TC3_RS[3:0] | Timers/Counters #3 interrupt route select | R/W | 0 |
| n.15-n.12 | TC2_RS[3:0] | Timers/Counters #2 interrupt route select | R/W | 0 |
| n.11-n.8 | TC1_RS[3:0] | Timers/Counters #1 interrupt route select | R/W | 0 |
| n.7-n.4 | TC0_RS[3:0] | Timers/Counters #0 interrupt route select | R/W | 0 |
| n.3-n.0 | GPIO 1_RS[3:0] | GPIO Port A,B,C,D interrupt route select. | R/W | 0 |

Table 22.  Interrupt Routing Register 5 (IRR5_1)   (0xB800_3124)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31-n.28 | GPIO 0_RS[3:0] | GPIO Port E,F,G,H interrupt route select. | R/W | 0 |
| n.27-n.24 | SWITCH_RS[3:0] | SWITCH interrupt route select | R/W | 0 |
| n.23-n.20 | SATA_RS[3:0] | SATA interrupt route select | R/W | 0 |
| n.19-n.16 | NFBI_RS[3:0] | NFBI interrupt route select | R/W | 0 |
| n.15-n.12 | WDOG_RS[3:0] | WDOG interrupt route select | R/W | 0 |
| n.11-n.8 | FFTACC_RS[3:0] | FFTACC interrupt route select | R/W | 0 |
| n.7-n.4 | FLSH_RS[3:0] | NAND FLSH interrupt route select | R/W | 0 |
| n.3-n.0 | UTMD_RS[3:0] | UTOPIA MDIO interrupt route select | R/W | 0 |

## Timer Control

The RTL8196 provides two sets of hardware timers and one watchdog timer. Each timer can be configured as timer mode or counter mode. Counter mode means the timer only times-out once. The initial time-out values are configured via TC0DATA and TC1DATA. The current count values are shown in TC0CNT and TC1CNT. CDBR defines the base clock for counting, which is based on a multiple of the system clock . TCIR controls the interrupt resulting from timer time-out. The Watchdog timer is controlled by WDTCNR.

Table 23.  Timer Control (Base = 0xB800-3200 )

| Offset | Size (byte) | Name | Description | Field |
| --- | --- | --- | --- | --- |
| 0x00 | 4 | TC0DATA | Timer/Counter 0 data register. It specifies the time-out duration. | MISC |
| 0x04 | 4 | TC1DATA | Timer/Counter 1 data register. It specifies the time-out duration. | MISC |
| 0x08 | 4 | TC0CNT | Timer/Counter 0 count register. | MISC |
| 0x0C | 4 | TC1CNT | Timer/Counter 1 count register. | MISC |
| 0x10 | 4 | TCCNR | Timer/Counter control register. | MISC |
| 0x14 | 4 | TCIR | Timer/Counter interrupt register. | MISC |
| 0x18 | 4 | CDBR | Clock division base register. | MISC |
| 0x1C | 4 | WDTCNR | Watchdog timer control register. | MISC |
| 0x20 | 4 | TC2DATA | Timer/Counter 2 data register. It specifies the time-out duration. | MISC |
| 0x24 | 4 | TC3DATA | Timer/Counter 3 data register. It specifies the time-out duration. | MISC |
| 0x28 | 4 | TC2CNT | Timer/Counter 2 count register. | MISC |
| 0x2C | 4 | TC3CNT | Timer/Counter 3 count register. | MISC |
| 0x30 | 4 | TC4DATA | Timer/Counter 4 data register. It specifies the time-out duration. | MISC |
| 0x34 | 4 | TC5DATA | Timer/Counter 5 data register. It specifies the time-out duration. | MISC |
| 0x38 | 4 | TC4CNT | Timer/Counter 4 count register. | MISC |
| 0x3C | 4 | TC5CNT | Timer/Counter 5 count register. | MISC |

Table 24.  Timer/Counter 0 Data Register    (offset=0x00)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31:4 | TC0Data[27:0] | The timer or counter initial value. 0 and 1 not allowed | R/W | 0H |

Table 25.  Timer/Counter 1 Data Register    (offset=0x04)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31:4 | TC1Data[27:0] | The timer or counter initial value 0 and 1 not allowed | R/W | 0H |

Table 26.  Timer/Counter 0 Counter Register    (offset=0x08)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31:4 | TC0Value[27:0] | The timer or counter value | R |  |

Table 27.  Timer/Counter 1 Counter Register    (offset=0x0c)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31:4 | TC1Value[27:0] | The timer or counter value | R |  |

Table 28.  Timer/Counter Control Register    (offset=0x10, byte access enable)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31 | TC0En | Timer/Counter 0 enable | R/W | 0 |
| 30 | TC0Mode | Timer/Counter 0 mode 0=Counter mode 1=Timer mode | R/W | 0 |
| 29 | TC1En | Timer/Counter 1 enable | R/W | 0 |
| 28 | TC1Mode | Timer/Counter 1 mode 0=counter mode 1=timer mode (When Mitigation&Timer1 is asserted, this bit should be set as 1 to assure normal processing) | R/W | 0 |
| 27 | TC2En | Timer/Counter 2 enable | R/W | 0 |
| 26 | TC2Mode | Timer/Counter 2 mode 0=Counter mode 1=Timer mode | R/W | 0 |
| 25 | TC3En | Timer/Counter 3 enable | R/W | 0 |
| 24 | TC3Mode | Timer/Counter 3 mode 0=counter mode 1=timer mode (When Mitigation&Timer1 is asserted, this bit should be set as 1 to assure normal processing) | R/W | 0 |
| 23 | TC4En | Timer/Counter 4 enable | R/W | 0 |
| 22 | TC4Mode | Timer/Counter 4 mode 0=Counter mode 1=Timer mode | R/W | 0 |
| 21 | TC5En | Timer/Counter 5 enable | R/W | 0 |
| 20 | TC5Mode | Timer/Counter 5 mode 0=counter mode 1=timer mode (When Mitigation&Timer1 is asserted, this bit should be set as 1 to assure normal processing) | R/W | 0 |

Table 29.  Timer/Counter Interrupt Register    (offset=0x14, byte access enable)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31 | TC0IE | Timer/Counter 0 interrupt enable. | R/W | 0 |
| 30 | TC1IE | Timer/Counter 1 interrupt enable (When Mitigation&Timer1 is asserted, this bit should be set as 0 to assure normal processing) | R/W | 0 |
| 29 | TC0IP | Timer/Counter 0 interrupt pending. Write ‘1’ to clear the interrupt. | R/W | 0 |
| 28 | TC1IP | Timer/Counter 1 interrupt pending. Write ‘1’ to clear the interrupt. | R/W | 0 |
| 27 | TC2IE | Timer/Counter 2 interrupt enable. | R/W | 0 |
| 26 | TC3IE | Timer/Counter 3 interrupt enable (When Mitigation&Timer1 is asserted, this bit should be set as 0 to assure normal processing) | R/W | 0 |
| 25 | TC2IP | Timer/Counter 2 interrupt pending. Write ‘1’ to clear the interrupt. | R/W | 0 |
| 24 | TC3IP | Timer/Counter 3 interrupt pending. Write ‘1’ to clear the interrupt. | R/W | 0 |
| 23 | TC4IE | Timer/Counter 4 interrupt enable. | R/W | 0 |
| 22 | TC5IE | Timer/Counter 5 interrupt enable (When Mitigation&Timer1 is asserted, this bit should be set as 0 to assure normal processing) | R/W | 0 |
| 21 | TC4IP | Timer/Counter 4 interrupt pending. Write ‘1’ to clear the interrupt. | R/W | 0 |
| 20 | TC5IP | Timer/Counter 5 interrupt pending. Write ‘1’ to clear the interrupt. | R/W | 0 |

Table 30.  Clock Division Base Register    (offset=0x18)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31:16 | DivFactor[16:0] | The divide factor of clock source. Assume DivFactor = N, then Base clock = System_clock /N. Both values 0x0000 and 0x0001 disable the clock. The CDBR register defined the clock base for Timer/Counter counting, which is based on the 1/2* system clock (Lexra bus clock) which is reflect on SCR of register MACMR. | R/W | 0x0000 |

Table 31.  Watchdog0 and Watchdog1 Control Register    (offset=0x1c, byte access enable)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31:24 | WDTE0[7:0] | Watchdog enable. When these bits are set to 0xA5, the watchdog timer stops. Other values will enable the watchdog timer and cause a system reset when an overflow signal occurs. | W | 0xA5 |
| 23 | WDTCLR0 | Watchdog clear. Write a 1 to clear the up-count watchdog counter. | W | 0 |
| 22:21 | OVSEL0[1:0] | Overflow select. These bits specify the overflow condition when the watchdog timer counts to the value. The watch dog timer is based on the base clock defined by CDBR. 00=215 01=216 10=217 11=218 | R/W | 00 |
| 20 | WatchDogIND0 | Watchdog Event Indicator. ‘0’: A Watchdog RESET did not occur (POWER-ON or PIN RESET) ‘1’: A Watchdog RESET occurred Write ‘1’ to clear. | R/W | 0 |
| 19 | NRFRstType0 | NOR Flash Reset Command Type selection When the watchdog event is active and WatchDogIND =1, It will cause the memory controller reboot and issue a Flash reset command. the command type should be pre-defined by this control bit. 0 = AMD NOR Flash reset command Type 1 = Intel NOR Flash reset command Type Design Note : This bit should not be reset by watch dog reset. | R/W | 0 |
| 18:17 | OVSEL0[3:2] | Higher Overflow Select bits. These bits specify the overflow condition when the watchdog timer counts to the value. The watch dog timer is based on the base clock defined by CDBR. 00: 20 x (the counter value of the Lower overflow select bits). 01: 24 x (the counter value of the Lower overflow select bits). 10: 28 x (the counter value of the Lower overflow select bits). 11: 212 x (the counter value of the Lower overflow select bits). The all condition value list: 0000: 215 0001: 216 0010: 217 0011: 218 0100: 219 0101: 220 0110: 221 0111: 222 1000: 223 1001: 224 | R/W | 00 |
| 15:8 | WDTE1[7:0] | Watchdog enable. When these bits are set to 0xA5, the watchdog timer stops. Other values will enable the watchdog timer and cause a system reset when an overflow signal occurs. | W | 0xA5 |
| 7 | WDTCLR1 | Watchdog clear. Write a 1 to clear the up-count watchdog counter. | W | 0 |
| 6:5 | OVSEL1[1:0] | Overflow select. These bits specify the overflow condition when the watchdog timer counts to the value. The watch dog timer is based on the base clock defined by CDBR. 00=215 01=216 10=217 11=218 | R/W | 00 |
| 4 | WatchDogIND1 | Watchdog Event Indicator. ‘0’: A Watchdog RESET did not occur (POWER-ON or PIN RESET) ‘1’: A Watchdog RESET occurred Write ‘1’ to clear. | R/W | 0 |
| 3 | NRFRstType1 | NOR Flash Reset Command Type selection When the watchdog event is active and WatchDogIND =1, It will cause the memory controller reboot and issue a Flash reset command. the command type should be pre-defined by this control bit. 0 = AMD NOR Flash reset command Type 1 = Intel NOR Flash reset command Type Design Note : This bit should not be reset by watch dog reset. | R/W | 0 |
| 2:1 | OVSEL1[3:2] | Higher Overflow Select bits. These bits specify the overflow condition when the watchdog timer counts to the value. The watch dog timer is based on the base clock defined by CDBR. 00: 20 x (the counter value of the Lower overflow select bits). 01: 24 x (the counter value of the Lower overflow select bits). 10: 28 x (the counter value of the Lower overflow select bits). 11: 212 x (the counter value of the Lower overflow select bits). The all condition value list: 0000: 215 0001: 216 0010: 217 0011: 218 0100: 219 0101: 220 0110: 221 0111: 222 1000: 223 1001: 224 | R/W | 00 |

Table 32  Timer/Counter 2 Data Register    (offset=0x20)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31:4 | TC2Data[27:0] | The timer or counter initial value. 0 and 1 not allowed | R/W | 0H |

Table 33  Timer/Counter 3 Data Register    (offset=024x)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31:4 | TC3Data[27:0] | The timer or counter initial value 0 and 1 not allowed | R/W | 0H |

Table 34  Timer/Counter 2 Counter Register    (offset=0x28)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31:4 | TC2Value[27:0] | The timer or counter value | R |  |

Table 35  Timer/Counter 3 Counter Register    (offset=0x2c)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31:4 | TC3Value[27:0] | The timer or counter value | R |  |

Table 36  Timer/Counter 4 Data Register    (offset=0x30)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31:4 | TC4Data[27:0] | The timer or counter initial value. 0 and 1 not allowed | R/W | 0H |

Table 37  Timer/Counter 5 Data Register    (offset=0x34)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31:4 | TC5Data[27:0] | The timer or counter initial value 0 and 1 not allowed | R/W | 0H |

Table 38  Timer/Counter 4 Counter Register    (offset=0x38)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31:4 | TC4Value[27:0] | The timer or counter value | R |  |

Table 39  Timer/Counter 5 Counter Register    (offset=0x3c)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31:4 | TC5Value[27:0] | The timer or counter value | R |  |

## GPIO Control

The RTL865xC provides 8 sets of General Purpose Input/Output (GPIO) pins (GPIO A,B,C,D,E,F,G,H) . Each GPIO pin may be configured as an input or output pin . The GPIO DATA register may be used to control the signals of GPIO pins. Because the GPIO pins are shared with some peripheral pins, The type of peripheral can also control the attributes of the shared pins. All GPIO sets can be used to generate interrupts and the interrupt mask and status register are provided. All the GPIO control register are defined as following table.

Table 40.  GPIO Register Set (Base = 0xB800-3300, all register support byte access)

| Offset | Size (byte) | Name | Description | Field |
| --- | --- | --- | --- | --- |
| 0x00 | 4 | PABCD_CNR | Port A, B, C, D control register. | MISC |
| 0x04 | 4 | PABCD_Ptype | Port A,B,C, D peripheral type control register. | MISC |
| 0x08 | 4 | PABCD_DIR | Port A, B, C, D direction register. | MISC |
| 0x0C | 4 | PABCD_DAT | Port A, B, C,D data register. | MISC |
| 0x10 | 4 | PABCD_ISR | Port A, B, C,D interrupt status register. | MISC |
| 0x14 | 4 | PAB_IMR | Port A, B interrupt mask register. | MISC |
| 0x18 | 4 | PCD_IMR | Port C, D interrupt mask register. | MISC |
| 0x1C | 4 | PEFGH_CNR | Port E, F, G, H control register. | MISC |
| 0x20 | 4 | PEFGH_Ptype | Port E, F, G, H peripheral type control register. | MISC |
| 0x24 | 4 | PEFGH_DIR | Port E, F, G, H direction register. | MISC |
| 0x28 | 4 | PEFGH_DAT | Port E, F, G, H data register. | MISC |
| 0x2C | 4 | PEFGH_ISR | Port E, F, G, H interrupt status register. | MISC |
| 0x30 | 4 | PEF_IMR | Port E, F interrupt mask register. | MISC |
| 0x34 | 4 | PGH_IMR | Port G, H interrupt mask register. | MISC |
| 0x38 | 4 | PABCD_CPU0_IMR | Port A, B, C, D cpu0 interrupt mask register. | MISC |
| 0x3C | 4 | PABCD_CPU1_IMR | Port A, B, C, D cpu1 interrupt mask register. | MISC |

Table 41.  GPIO Port A, B, C, D Control Register (PABCD_CNR)   (0x00)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31:24 | PFC_D[7:0] | Pin function configuration of Port D. | R/W | FFH |
| 23:16 | PFC_C[7:0] | Pin function configuration of Port C. | R/W | FFH |
| 15:8 | PFC_B[7:0] | Pin function configuration of Port B. | R/W | FFH |
| 7:0 | PFC_A[7:0] | Pin function configuration of Port A. Bit value : 0=Configured as GPIO pin 1=Configured as dedicated peripheral pin | R/W | FFH |

Table 42.  GPIO Port A, B, C,D Direction Register (PABCD_DIR)   (0x08)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31-n.24 | DRC_D[7:0] | Pin direction configuration of Port D. 0=Configured as input pin 1=Configured as output pin | R/W | 00H |
| n.23-n.16 | DRC_C[7:0] | Pin direction configuration of Port C. 0=Configured as input pin 1=Configured as output pin | R/W | 00H |
| n.15-n.8 | DRC_B[7:0] | Pin direction configuration of Port B. 0=Configured as input pin 1=Configured as output pin | R/W | 00H |
| n.7-n.0 | DRC_A[7:0] | Pin direction configuration of Port A. 0=Configured as input pin 1=Configured as output pin | R/W | 00H |

Table 43.  Port A, B,C,D Data Register (PABCD_DAT)   (0x0C)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31-n.24 | PD_D[7:0] | Pin data of Port D. 0 : Data=0 1 : Data=1 | R, R/W | 00H |
| n.23-n.16 | PD_C[7:0] | Pin data of Port C. 0 : Data=0 1 : Data=1 | R, R/W | 00H |
| n.15-n.8 | PD_B[7:0] | Pin data of Port B. 0 : Data=0 1 : Data=1 | R, R/W | 00H |
| n.7-n.0 | PD_A[7:0] | Pin data of Port A. 0 : Data=0 1 : Data=1 | R, R/W | 00H |

Table 44.  Port A, B,C,D Interrupt Status Register (PABCD_ISR)   (0x10)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31-n.24 | IPS_D[7:0] | Interrupt pending status of port D. Write ‘1’ to clear the interrupt | R/W | 00H |
| n.23-n.16 | IPS_C[7:0] | Interrupt pending status of port C. Write ‘1’ to clear the interrupt | R/W | 00H |
| n.15-n.8 | IPS_B[7:0] | Interrupt pending status of port B. Write ‘1’ to clear the interrupt | R/W | 00H |
| n.7-n.0 | IPS_A[7:0] | Interrupt pending status of port A. Write ‘1’ to clear the interrupt | R/W | 00H |

Table 45.  Port A, B Interrupt Mask Register (PAB_IMR)   (0x14)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31- n.30 | PB7_IM[1:0] | PortB.7 interrupt mode. | R/W | 00B |
| n.29- n.28 | PB6_IM[1:0] | PortB.6 interrupt mode. | R/W | 00B |
| n.27- n.26 | PB5_IM[1:0] | PortB.5 interrupt mode. | R/W | 00B |
| n.25- n.24 | PB4_IM[1:0] | PortB.4 interrupt mode. | R/W | 00B |
| n.23- n.22 | PB3_IM[1:0] | PortB.3 interrupt mode. | R/W | 00B |
| n.21- n.20 | PB2_IM[1:0] | PortB.2 interrupt mode. | R/W | 00B |
| n.19- n.18 | PB1_IM[1:0] | PortB.1 interrupt mode. | R/W | 00B |
| n.17- n.16 | PB0_IM[1:0] | PortB.0 interrupt mode. | R/W | 00B |
| n.15- n.14 | PA7_IM[1:0] | PortA.7 interrupt mode. | R/W | 00B |
| n.13-n.12 | PA6_IM[1:0] | PortA.6 interrupt mode. | R/W | 00B |
| n.11- n.10 | PA5_IM[1:0] | PortA.5 interrupt mode. | R/W | 00B |
| n.9-n.8 | PA4_IM[1:0] | PortA.4 interrupt mode. | R/W | 00B |
| n.7-n.6 | PA3_IM[1:0] | PortA.3 interrupt mode. | R/W | 00B |
| n.5-n.4 | PA2_IM[1:0] | PortA.2 interrupt mode. | R/W | 00B |
| n.3-n.2 | PA1_IM[1:0] | PortA.1 interrupt mode. | R/W | 00B |
| n.1-n.0 | PA0_IM[1:0] | PortA.0 interrupt mode. 00=Disable interrupt 01=Enable falling edge interrupt 10=Enable rising edge interrupt 11=Enable both falling or rising edge interrupt | R/W | 00B |

Table 46.  Port C, D Interrupt Mask Register (PCD_IMR)   (0x18)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31- n.30 | PD7_IM[1:0] | PortD.7 interrupt mode. | R/W | 00B |
| n.29- n.28 | PD6_IM[1:0] | PortD.6 interrupt mode. | R/W | 00B |
| n.27- n.26 | PD5_IM[1:0] | PortD.5 interrupt mode. | R/W | 00B |
| n.25- n.24 | PD4_IM[1:0] | PortD.4 interrupt mode. | R/W | 00B |
| n.23- n.22 | PD3_IM[1:0] | PortD.3 interrupt mode. | R/W | 00B |
| n.21- n.20 | PD2_IM[1:0] | PortD.2 interrupt mode. | R/W | 00B |
| n.19- n.18 | PD1_IM[1:0] | PortD.1 interrupt mode. | R/W | 00B |
| n.17- n.16 | PD0_IM[1:0] | PortC.0 interrupt mode. | R/W | 00B |
| n.15- n.14 | PC7_IM[1:0] | PortC.7 interrupt mode. | R/W | 00B |
| n.13-n.12 | PC6_IM[1:0] | PortC.6 interrupt mode. | R/W | 00B |
| n.11- n.10 | PC5_IM[1:0] | PortC.5 interrupt mode. | R/W | 00B |
| n.9-n.8 | PC4_IM[1:0] | PortC.4 interrupt mode. | R/W | 00B |
| n.7-n.6 | PC3_IM[1:0] | PortC.3 interrupt mode. | R/W | 00B |
| n.5-n.4 | PC2_IM[1:0] | PortC.2 interrupt mode. | R/W | 00B |
| n.3-n.2 | PC1_IM[1:0] | PortC.1 interrupt mode. | R/W | 00B |
| n.1-n.0 | PC0_IM[1:0] | PortC.0 interrupt mode. 00=Disable interrupt 01=Enable falling edge interrupt 10=Enable rising edge interrupt 11=Enable both falling or rising edge interrupt | R/W | 00B |

Table 47.  GPIO Port E, F, G, H Control Register (PEFGH_CNR)   (0x1C)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 31:24 | PFC_H[7:0] | Pin function configuration of Port H. | R/W | FFH |
| 23:16 | PFC_G[7:0] | Pin function configuration of Port G. | R/W | FFH |
| 15:8 | PFC_F[7:0] | Pin function configuration of Port F. | R/W | FFH |
| 7:0 | PFC_E[7:0] | Pin function configuration of Port E. Bit value : 0=Configured as GPIO pin 1=Configured as dedicated peripheral pin | R/W | FFH |

Table 48.  GPIO Port E, F, G, H Direction Register (PEFGH_DIR)   (0x24)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31-n.24 | DRC_H[7:0] | Pin direction configuration of Port H. 0=Configured as input pin 1=Configured as output pin | R/W | 00H |
| n.23-n.16 | DRC_G[7:0] | Pin direction configuration of Port G. 0=Configured as input pin 1=Configured as output pin | R/W | 00H |
| n.15-n.8 | DRC_F[7:0] | Pin direction configuration of Port F. 0=Configured as input pin 1=Configured as output pin | R/W | 00H |
| n.7-n.0 | DRC_E[7:0] | Pin direction configuration of Port E. 0=Configured as input pin 1=Configured as output pin | R/W | 00H |

Table 49.  Port E, F, G, H Data Register (PEFGH_DAT)   (0x28)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31-n.24 | PD_H[7:0] | Pin data of Port H. 0 : Data=0 1 : Data=1 | R, R/W | 00H |
| n.23-n.16 | PD_G[7:0] | Pin data of Port G. 0 : Data=0 1 : Data=1 | R, R/W | 00H |
| n.15-n.8 | PD_F[7:0] | Pin data of Port F. 0 : Data=0 1 : Data=1 | R, R/W | 00H |
| n.7-n.0 | PD_E[7:0] | Pin data of Port E. 0 : Data=0 1 : Data=1 | R, R/W | 00H |

Table 50.  Port E, F, G, H Interrupt Status Register (PEFGH_ISR)   (0x2C)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31-n.24 | IPS_H[7:0] | Interrupt pending status of port H. Write ‘1’ to clear the interrupt | R/W | 00H |
| n.23-n.16 | IPS_G[7:0] | Interrupt pending status of port G. Write ‘1’ to clear the interrupt | R/W | 00H |
| n.15-n.8 | IPS_F[7:0] | Interrupt pending status of port F. Write ‘1’ to clear the interrupt | R/W | 00H |
| n.7-n.0 | IPS_E[7:0] | Interrupt pending status of port E. Write ‘1’ to clear the interrupt | R/W | 00H |

Table 51.  Port E, F Interrupt Mask Register (PEF_IMR)   (0x30)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31- n.30 | PF7_IM[1:0] | PortF.7 interrupt mode. | R/W | 00B |
| n.29- n.28 | PF6_IM[1:0] | PortF.6 interrupt mode. | R/W | 00B |
| n.27- n.26 | PF5_IM[1:0] | PortF.5 interrupt mode. | R/W | 00B |
| n.25- n.24 | PF4_IM[1:0] | PortF.4 interrupt mode. | R/W | 00B |
| n.23- n.22 | PF3_IM[1:0] | PortF.3 interrupt mode. | R/W | 00B |
| n.21- n.20 | PF2_IM[1:0] | PortF.2 interrupt mode. | R/W | 00B |
| n.19- n.18 | PF1_IM[1:0] | PortF.1 interrupt mode. | R/W | 00B |
| n.17- n.16 | PF0_IM[1:0] | PortF.0 interrupt mode. | R/W | 00B |
| n.15- n.14 | PE7_IM[1:0] | PortE.7 interrupt mode. | R/W | 00B |
| n.13-n.12 | PE6_IM[1:0] | PortE.6 interrupt mode. | R/W | 00B |
| n.11- n.10 | PE5_IM[1:0] | PortE.5 interrupt mode. | R/W | 00B |
| n.9-n.8 | PE4_IM[1:0] | PortE.4 interrupt mode. | R/W | 00B |
| n.7-n.6 | PE3_IM[1:0] | PortE.3 interrupt mode. | R/W | 00B |
| n.5-n.4 | PE2_IM[1:0] | PortE.2 interrupt mode. | R/W | 00B |
| n.3-n.2 | PE1_IM[1:0] | PortE.1 interrupt mode. | R/W | 00B |
| n.1-n.0 | PE0_IM[1:0] | PortE.0 interrupt mode. 00=Disable interrupt 01=Enable falling edge interrupt 10=Enable rising edge interrupt 11=Enable both falling or rising edge interrupt | R/W | 00B |

Table 52.  Port C, D Interrupt Mask Register (PCD_IMR)   (0x34)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31- n.30 | PH7_IM[1:0] | PortH.7 interrupt mode. | R/W | 00B |
| n.29- n.28 | PH6_IM[1:0] | PortH.6 interrupt mode. | R/W | 00B |
| n.27- n.26 | PH5_IM[1:0] | PortH.5 interrupt mode. | R/W | 00B |
| n.25- n.24 | PH4_IM[1:0] | PortH.4 interrupt mode. | R/W | 00B |
| n.23- n.22 | PH3_IM[1:0] | PortH.3 interrupt mode. | R/W | 00B |
| n.21- n.20 | PH2_IM[1:0] | PortH.2 interrupt mode. | R/W | 00B |
| n.19- n.18 | PH1_IM[1:0] | PortH.1 interrupt mode. | R/W | 00B |
| n.17- n.16 | PH0_IM[1:0] | PortH.0 interrupt mode. | R/W | 00B |
| n.15- n.14 | PG7_IM[1:0] | PortG.7 interrupt mode. | R/W | 00B |
| n.13-n.12 | PG6_IM[1:0] | PortG.6 interrupt mode. | R/W | 00B |
| n.11- n.10 | PG5_IM[1:0] | PortG.5 interrupt mode. | R/W | 00B |
| n.9-n.8 | PG4_IM[1:0] | PortG.4 interrupt mode. | R/W | 00B |
| n.7-n.6 | PG3_IM[1:0] | PortG.3 interrupt mode. | R/W | 00B |
| n.5-n.4 | PG2_IM[1:0] | PortG.2 interrupt mode. | R/W | 00B |
| n.3-n.2 | PG1_IM[1:0] | PortG.1 interrupt mode. | R/W | 00B |
| n.1-n.0 | PG0_IM[1:0] | PortG.0 interrupt mode. 00=Disable interrupt 01=Enable falling edge interrupt 10=Enable rising edge interrupt 11=Enable both falling or rising edge interrupt | R/W | 00B |

Table 53.  Port A, B, C, D CPU0 Interrupt Mask Register (PABCD_CPU0_IMR)   (0x38)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31- n.24 | PD_CPU0_IM[7:0] | PortD interrupt mask for CPU0. | R/W | 00B |
| n.23- n.16 | PC_CPU0_IM[7:0] | PortC interrupt mask for CPU0. | R/W | 00B |
| n.15- n.8 | PB_CPU0_IM[7:0] | PortB interrupt mask for CPU0. | R/W | 00B |
| n.7- n.0 | PA_CPU0_IM[7:0] | PortA interrupt mask for CPU0. 0 = disable 1 = enable | R/W | 00B |

Table 54.  Port A, B, C, D CPU1 Interrupt Mask Register (PABCD_CPU1_IMR)   (0x3C)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| n.31- n.24 | PD_CPU1_IM[7:0] | PortD interrupt mask for CPU1. | R/W | 00B |
| n.23- n.16 | PC_CPU1_IM[7:0] | PortC interrupt mask for CPU1. | R/W | 00B |
| n.15- n.8 | PB_CPU1_IM[7:0] | PortB interrupt mask for CPU1. | R/W | 00B |
| n.7- n.0 | PA_CPU1_IM[7:0] | PortA interrupt mask for CPU1. 0 = disable 1 = enable | R/W | 00B |