Annex D (normative):
DFS parameters

Table D.1: DFS requirement values

| Parameter | Value |
| --- | --- |
| Channel Availability Check Time | 60 s |
| Channel Move Time | 10 s |
| Channel Closing Transmission Time | 260 ms |
| Non-Occupancy Period | 30 min |

Table D.2: Interference Threshold values, Master

| Maximum Transmit Power (eirp) | Value (see note) |
| --- | --- |
| 200 mW | -64 dBm |
| 200 mW | -62 dBm |
| NOTE: This is the level at the input of the receiver assuming a 0 dBi receive antenna. |  |

Table D.3: Interference Threshold values, Slave

| Maximum Transmit Power (eirp) | Value (see note) |
| --- | --- |
| 200 mW | -64 dBm |
| 200 mW | N/A |
| NOTE: This is the level at the input of the receiver assuming a 0 dBi receive antenna. |  |

Table D.4: Parameters of DFS test signals

| Radar test signal | Pulse width W [µs] (see Note 6) choose 1 | Pulse repetition frequency PRF [pps] choose 1 | Pulses per burst (note 1) | Detection probability with 30% channel load |
| --- | --- | --- | --- | --- |
| 1 - Fixed | 1 | 750 | 15 | Pd > 60 % |
| 2 - Variable | 1, 2, 5 | 200, 300, 500, 800, 1000 | 10 | Pd > 60 % |
| 3 - Variable | 10, 15 | 200, 300, 500, 800, 1000 | 15 | Pd > 60 % |
| 4 - Variable | 1, 2, 5, 10, 15 | 1200, 1500, 1600 | 15 | Pd > 60 % |
| 5 - Variable | 1, 2, 5, 10, 15 | 2300, 3000, 3500, 4000 | 25 | Pd > 60 % |
| 6 – Variable modulated (note 6) | 20, 30 | 2000, 3000, 4000 | 20 | Pd > 60 % |
| NOTE 1: This represents the number of pulses seen at the RLAN per radar scan: N = [{antenna beamwidth (deg)} x {pulse repetition rate (pps)}] / [{scan rate (deg/s)}]. NOTE 2: The test signals above only contain a single burst of pulses. NOTE 3: The number of pulses per burst given in this table simulate real radar systems and take into account the effects of pulse repetition rate and pulse width on the detection probability for a single burst. NOTE 4: Pd gives the probability of detection per simulated radar burst and represents a mimimum level of detection performance under defined conditions - in this case a 30% traffic load. Therefore Pd does not represent the overall detection probability for any particular radar under real life conditions. In general 5 sequential bursts are needed to achieve a real llife detection rate of better that 99% for any radar that falls within the scope of the above table. NOTE 5: The pulse width used in these tests is assumed to be representative of real radar systems with different pulse widths and different modulations. For example, a wide pulse that is frequency modulated will appear to the RLAN receiver as a much narrower pulse [NOTE 6: The modulation to be used for the radar test signal 6 is a chirp modulation with a ± 2,5MHz frequency deviation which is described below. ] |  |  |  |  |
| ` |  |  |  |  |