5 GHz DFS Status

6/2/05

Test Signals

The 5 GHz Project Team has made excellent progress on completing the work required to finalize its recommendations on the DFS conformance test procedures.  The cooperation between NTIA, FCC, DoD, NASA and industry has led to the development of agreed upon test signals (See figure 1, 2 and 3).

Figure 1 - Fixed System Radars

| Fixed Radar Set | Pulse Width (usec) | PRI (usec) | Pulses | Minimum Probability of Detection | Minimum Trials |
| --- | --- | --- | --- | --- | --- |
| 1 - fixed | 1 | 1428 | 18 | 60% | 10 |
| 2 - variable | 1-5 | 150-230 | 23-29 | 60% | 10 |
| 3 - variable | 6-10 | 200-500 | 16-18 | 60% | 10 |
| 4 - variable | 11-20 | 200-500 | 12-16 | 50% | 10 |

The test signal parameters shown in Figure 1 represent the first set of test signals to be used to perform the conformance test procedures.  The values shown in the table represent a consensus of all participants in the 5 GHz Project Team. The agreement includes that a minimum of 10 trials per set be run with a minimum probability of detection (as shown in figure 1) calculated by:

Forumula 1:  	TotalSetDetectionsTotalSetTrials×100

In addition the agreement includes that an average minimum probability of 85% must be met across all four radar sets and is calculated as follows:

Forumula 2:  	TotalDetectionsTotalTrials×100

Agreement was also reached on the step values for each of the variable radar parameters shown in Figure 1.  A step interval of .1 usec was agreed for Pulse Width, a 1 usec step interval was agreed for PRI, and a step interval of 1 was agreed for the number of pulses.

Figure 2 – Long Pulse Radars

| Long Pulse Radar Set | Pulse Width (usec) | PRI (usec) | Pulses | Minimum probability of detection | Minimum Trials |
| --- | --- | --- | --- | --- | --- |
| 1 | 50-100 | 1000-5000 | 1-3 | [90%] | [10] |

The test signal parameters shown in Figure 2 represent the second set of test signals to be performed and will test for conformance against long pulse radars.  The test signal parameters in the table represent consensus of all participants in the 5 GHz Project Team.  The specific number of trials and the number of successful events detected to show compliance is still under discussion by the project team.  The value proposed by the Federal Government is shown in Figure 2 in square brackets.  The proposal includes that a minimum of 10 trials per set be run with a minimum probability of detection (as shown in figure 1) calculated by:

Forumula 3:  	TotalSetDetectionsTotalSetTrials×100

NTIA, DoD and FCC have discussed the alternative method using time averaging provided by industry.  At the present time we do not believe that calculating the average provides sufficient evidence of compliance with the requirement for minimum probability of detection and hence, may not provide devices that will sufficiently protect the Government radar systems.  Without more information about the performance of the devices in real operation, and given recent discussion that led to the discovery that industry devices cannot detect all single pulse test signals, further progress on defining the number of successful events per trial of 10 or more will be difficult if not impossible.  Therefore, discussion about the finalization of specific parameters for final compliance purposes has been tabled until after the NTIA has completed the baseline/field testing.  Based on the detailed performance information gathered during those tests, the Federal Government will be in a better position to finalize the requirements and will be able to address industry concerns at that time.  Upon completion of baseline and field tests, the Federal Government will review with the FCC and industry to determine if there is any need to change the current deadlines for application for new certification in the 5250-5350 MHz band.

Agreement was also reached on the step values for each of the variable radar parameters shown in Figure 2.  A step interval of .1 usec was agreed for Pulse Width, a 1 usec step interval was agreed for PRI, and a step interval of 1 was agreed for the number of pulses.

Figure 3 – Frequency Hopping Radars

| Parameter Type | Pulse Width (sec) | PRI (sec) | Pulses per Hop | Pass/Fail | Minimum Trials |
| --- | --- | --- | --- | --- | --- |
| 1 - Fixed | 1 | 333 | 9 | 70% | 10 |

The test signal parameters shown in Figure 3 represent the last set of test signals to be used to perform the conformance test procedures.  The values shown in the table represent a consensus of all participants in the 5 GHz Project Team. The agreement includes that a minimum of 10 trials per set be run with a minimum probability of detection (as shown in figure 1) calculated by:

Forumula 4:  	TotalSetDetectionsTotalSetTrials×100

Agreement was also reached on the step values for each of the variable radar parameters shown in Figure 2.  A step interval of .1 usec was agreed for Pulse Width, a 1 usec step interval was agreed for PRI, and a step interval of 1 was agreed for the number of pulses.

Test Procedures

The 5 GHz Project Team is still actively working to resolve the test procedures themselves.  The Federal Government has been asked to review the requirements it has set on testing to address the following points:

- Can a test mode be utilized to facilitate testing of larger data sets on each bin?  NTIA and DoD cannot agree to this until a comparison is made between test modes and operational modes during the NTIA baseline/field testing.

- Is there a need to test for channel move time after every trial?  NTIA and DoD are discussing the possibility of developing a separate test for each bin that would determine if the device met the required channel move time and then performing a detection probability test that would not include measurement of channel move time.  This will also be reviewed during the NTIA baseline/field testing.

- Is there a need to define the minimum detection bandwidth and if so, what minimum would be required?  This is being reviewed by the Federal Government representatives and a decision should be forthcoming shortly.  In two separate telconferences this issue was discussed and as a group support providing such a definition.

- What alternate test procedures can be approved in addition to the preferred Government test set up?  DFS/18 is being revised to include one alternate test procedure.  DFS/24 also contains an alternate test procedure but it still requires a great deal of work.  Whatever alternate test procedures are agreed upon when the final version of DFS/2 is completed will be sent to the FCC as approved alternate test procedures recommended for incorporation.

Baseline/Field Testing

The dates for the baseline testing have been set as follows:

August 1 – 5

August 8-12

August 22-26

August 29 – September 02

Four Manufacturers have agreed to provide devices.  Each manufacturer is asked to provide 2 APs and a total of 10 clients to facilitate the testing (or 2 full configurations if they will be testing an atypical RLAN system).  Laptop computers for the client devices to run in would be greatly appreciated.  Manufacturers can have technicians present during this testing and they will be able to actively participate in the setup of their equipment and given a short trial period to work out any “kinks”.  Once testing has begun the device under test cannot be modified further and will be retained by NTIA until after the field testing.  The results of the baseline testing cannot be used for submission to the FCC for certification.  Each manufacturer will be given a dedicated week for testing and only technicians from the company under test will be present during that week.

The Field testing is currently being scheduled for Oct.  NTIA is working with DoD to ensure that one scanning radar and one tracking radar are present during the testing.  Testing will consist of separate tests for each bin waveform to compare detection results with those found during the baseline testing for the first 5 bins.  In addition the devices will be tested against several operational scenarios that will indicate the devices ability to detect against real-life conditions. As soon as the dates have been confirmed NTIA will notify the Project Team.  Manufacturers that are participating in the test can work with the Army to have representatives with the appropriate security clearance to be granted access to the test.  This is contingent on Range rules which are being investigated.