RTL8198 IMEM DMEM DMA control registers

Table 1 IMEM DMEM DMA Control Register Address Mapping

(Base Address = 0xB800-B800.)

| Offset | Size (byte) | Name | Description |
| --- | --- | --- | --- |
| 00 | 4 | IMEM_DMEM_SA | IMEM_DMEM start address of transfer |
| 04 | 4 | EXTM_SA | External memory start address of transfer |
| 08 | 4 | DMDMA_CTL | IMEM_DMEM DMA Control |

Base Address = 0xB800-B800.

IMEM_DMEM start address of transfer (start_imem_dmem_adr)

| Bit | Type | Function | Initial |
| --- | --- | --- | --- |
| 17:0 | R/W | Start (word)address of IMEM_DMEM for transfer (virtual address[19:2]) | 0 |

External memory start address of transfer (start_ddr_adr)

| Bit | Type | Function | Initial |
| --- | --- | --- | --- |
| 31:2 | R/W | Start (word)address of external memory for transfer (physical address) | 0 |

DMEM DMA Control

| Bit | Type | Function | Initial |
| --- | --- | --- | --- |
| 31:26 |  | Reserved |  |
| 25:23 | R/W | Imem/Dmem/L2mem selection, 000: imem0, 001: imem1, 010: dmem0, 011: dmem1, 100: L2mem | 0 |
| 22 | R/W | Acceleration mode (default is store and forward mode) | 0 |
| 21 | R/W | (soft_rst), set this bit to reset IMEM_DMEM DMA controller | 0 |
| 20 | R/C | (dma_start), set this bit to start DMA. Clear by DMA transfer is done | 0 |
| 19 | R/W | (direction), 0: IMEM_DMEM to DRAM 1: DRAM to IMEM_DMEM | 0 |
| 18:0 | R/W | Transfer size (tran_size) in words, 0x00001 to 0x40000 represents 1 to 2^18 words, 0x0000 and 0x40001 to 0x7ffff is NOT allowed | 0 |

P.S. IMEM0/IMEM1/DMEM0/DMEM1 baseaddr must be re-defined in header file.