TEW-653AP Firmware Upgrade Procedures

- Save the unzipped files to your local PC

- Login to configuration page via browser (default IP 192.168.10.100)

- Go to Tools tab on the left panel and select Firmware option

- Click on Browse button on Firmware page and find the unzipped Kernel file (TEW-653AP-KNL-xxxxxx) and click Apply.  Router will start to load the kernel file.

- After it’s complete, click on Browse button again and find the unzipped firmware file (TEW-653AP-APPS-xxxxxx.dlf) and click Apply. Router will start to load the new firmware and restart.

- Firmware upgrade complete.