108Mbps 802.11a+g Wireless Firewall Router

The TRENDnet TEW-511BRP Dual-Band 802.11a+g Wireless Router combines features from a wired router and wireless access point to create an all-in-one network solution for home or small office.  Wireless clients equipped with the TEW-501PC 802.11a/g adapter can take advantage of Super AG technology by using frame bursts, fast frames, frame compression, and dynamic turbo to reach speeds up to 108Mbps.  A built-in 4-port switch connects wired workstations to your network while the access point connects standard 802.11a/b/g wireless clients. TEW-511BRP is an ideal appliance for sharing Internet access and network resources. The TEW-511BRP’s firewall protects your network from hackers while WEP and WPA encryptions safeguard wireless transmission.

TEW-511BRP

Features

- Wi-Fi Compliant with IEEE 802.11a, 802.11g and IEEE 802.11b Standards

- Uses 5 GHz or 2.4 GHz Dual Band Design, which Complies with Worldwide Requirements

- 4 x 10/100Mbps Auto-MDIX LAN Port and 1 x 10/100Mbps WAN Port (Internet)

- Supports Cable/DSL Modems with Dynamic IP, Static IP, PPPoE, or PPTP Connection Types

- Supports Super G and Super AG and XR (eXtended Range) technology Data Rate up to 108Mbps (AP Mode Only)

- DHCP Server Feature Allocates up to 253 Client IP Addresses

- 64/128-bit WEP, WPA (Pre-Shared Key or RADIUS) supports TKIP or AES Algorithm

- Supports MAC Address, IP and Protocol Filters

- Firewall features Network Address Translation (NAT), and Stateful Packet Inspection (SPI) protects against Dos attacks

- Traffic Control with Virtual Server and DMZ

- Provides Additional Security of Enable/Disable SSID, Password Protection

- Supports Static/Dynamic Routing and Dynamic DNS

- Supports Multiple and Concurrent IPSec, L2TP and  PPTP VPN Pass-Through Sessions

- Flash Memory for Firmware Upgrade, Save/Restore Settings, and Traffic Log

- Easy Management via Web Browser (HTTP) and Remote Management

- Compliant with Windows 95/98/NT/2000/XP/2003 Server, Linux and Mac OS

- Range of 30 ~ 100 meters Indoor(depends on the environment)

- Range of 50 ~ 300 meters Outdoor(depends on the environment)

- 3-Year Warranty

Wireless A+G Networking Solution

* Due to European Wireless Regulations, Super A mode is not supported in the EU version.

Specification

| ### Hardware |  |
| --- | --- |
| ### Standards | ### Wired: IEEE 802.3 (10Base-T), IEEE 802.3u (100Base-TX) ### Wireless: IEEE 802.11a (54Mbps), 802.11b (11Mbps), IEEE 802.11g (54Mbps) |
| ### WAN | ### 1 x 10/100Mbps Auto-MDIX Port (Internet) |
| ### LAN | ### 4 x 10/100Mbps Auto-MDIX Port |
| ### Connection Type | ### Dynamic IP, Static (Fixed) IP, PPPoE, PPTP |
| ### Advance Features | ### DHCP Client/Server, SPI, NAT, PPPoE Client, DNS Proxy/Relay, Dynamic DNS DMZ Hosting, VPN Pass-Through (IPSec, PPTP, L2TP), NTP UPnP, NAT Traversal |
| ### Protocol Support | TCP/IP, NetBEUI, IPX/SPX |
| ### LED Indicator | ### Power, DMZ, 11a, 11g, LAN, Internet |
| ### Power Adapter | ### 5V DC, 2.5A External Power Adapter |
| ### Dimension (L x W x H) | ### 198 x 160 x 33 mm (7.8 x 6.3 x 1.2inches) |
| ### Weight | ### 0.7kg (1.54 lbs) |
| ### Temperature | ### Operation: 0~ 40C (32F~ 104F) Storage: -20~ 70C (-4F~ 158F) |
| ### Humidity | ### Max. 90% (Non-Condensing) |
| ### Certification | ### FCC & CE |
| ### Wireless |  |
| Module Technique | 802.11b: CCK (11 and 5.5Mbps), DQPSK (2Mbps), DBPSK (1Mbps) 802.11a/g: OFDM with BPSK, QPSK and 16/64-QAM Sub-Carrier Modulations |
| Frequency | 802.11a: UNI-Band 5.15 ~ 5.825 GHz (subject to local regulations) 802.11b/g: ISM-Band 2.412 ~ 2.4835 GHz |
| Antenna | 802.11a: 4dBi (Peak Gain); 802.11b/g: 2dBi (Peak Gain) 2 x Dual Band Diversity Detachable Antenna (Female Reverse SMA Connector) |
| Media Access Protocol | CSMA/CA with ACK |
| Data Rate | 802.11b: 11Mbps, 5.5Mbps, 2Mbps, and 1Mbps 802.11g: 54Mbps, 48Mbps, 36Mbps, 24Mbps, 18Mbps, 12Mbps, 9Mbps and 6Mbps Super AG: 108Mbps |
| ### Security | ### 64/128-bit WEP & WPA ### 802.1X (EAP-MD5/TLS/TTLS) ### SSID Broadcast Control, MAC Filter, IP Filter, Content Filter Time-of-Day ACL |
| ### Transmit Power | ### 802.11a: 16dBm (max) @ 16QAM R3/4 ### 802.11b: 18dBm (typical) ### 802.11g: 16dBm (typical) |
| Receiving Sensitivity (802.11a/g) | 108Mbps 10 – 5 BER @ -69 dBm (Turbo Mode) 54Mbps 10 – 5 BER @ -72 dBm ; 48Mbps 10 – 5 BER @ -75 dBm 36Mbps 10 – 5 BER @ -82 dBm ; 24Mbps 10 – 5 BER @ -85 dBm 18Mbps 10 – 5 BER @ -88 dBm ; 12Mbps 10 – 5 BER @ -89 dBm 9Mbps 10 – 5 BER @ -90 dBm ; 6Mbps 10 – 5 BER @ -91 dBm |
| Channels | 11a: Subject to Local Regulations 11b/g: 1~ 11 (US), 1~13 (EU) |

Package Contents				Related Products

- TEW-511BRP				- TEW-501PC

- Multi-Language QIG			- TEW-503PI

- CD-ROM (User’s Guide)			- TEW-504UB

- Power Adapter (5V DC, 2.5A)		- TEW-510APB

- RJ-45 Cable (1.5m)

To order please call: 888-326-6061

3135 Kashiwa Street • Torrance, CA 90505 • (310) 891-1100 • Fax: (310) 891-1111

Sales: sales@trendware.com • Support: techsupport@trendware.com

www.TRENDnet.com