19 dBi Outdoor 
High-Gain Directional Antenna

TEW-AO19D Outdoor Directional Antenna provides long distance point-to-point connectivity for your wireless network. As an optional antenna for your IEEE 802.11b/g 2.4GHz wireless network equipment, you can install it outdoor for long distance point-to-point connections. Enjoy the benefits of wireless connectivity at greater distances and throughput throughout your outdoor wireless zone.

* This dBi gain does not factor cable loss. Performance estimations are for typical use; range will vary with site conditions.

TEW-AO19D version V1.0R

Features

- Compatible with 2.4GHz 802.11b/g wireless devices

- Weatherproof housing suitable for outdoor applications

- Adjustable mount for optimal orientation and performance

- Ideal for point-to-point bridge connections between buildings/zones

- Enhances wireless range and improves signal transmission and reception*

- Boosts standard antenna power from a meager 2dBi to a respectable 19dBi

- Easy-to-install wall and pole mounting

- 3-Year Warranty

*Due to the bisymmetrical nature of WiFi, the client segment may also require a high gain antenna.

Specifications

| Hardware |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Frequency range | 2.4 ~ 2.5GHz |  | Impedance |  | 50 Ohms |
| Gain | 19dBi |  | Connector |  | N-type Female |
| VSWR | 1.5:1 Max |  | Survival wind speed |  | 201.2km/ hr (125mi./hr) |
| Polarization | Linear, vertical |  | Lighting Protection |  | DC Ground |
| HPBW/ Horizontal | 18° |  | Radoms Material |  | ABS |
| HPBW/ Vertical | 18° |  | Dimension |  | 390 x 390 x 30mm (15.4 x 15.4x 1.2in.) |
| Front to Back Ratio | 30dB |  | Weight |  | 950g (33.5oz.) |
| Power handling | 30W(cw) |  | Temperature |  | -40oC ~ 80oC (-40oF ~ 176oF) |

* Note: For optimal results in extending your wireless range with wireless antenna installations, we recommend that you consult a professional installer for site survey, safety precautions, and proper installation.

Patterns

Antenna Solution with TEW-AO19D

Package Contents							Related Products

- TEW-AO19D							- TEW-AO09D

- Multi-Language Quick Installation Guide				- TEW-AO14D

- Mounting Guide							- TEW-AO08O

- Mounting Brackets							- TEW-AO10O

To order please call: 888-326-6061; Sales: sales@trendnet.com

Support: support@trendnet.com; Tech Support Hotline: 866-845-3673

Official Site: www.trendnet.com