USB 2.0 to SATA / IDE Converter Adapter

The USB 2.0 to SATA/IDE combo adapter allows users to use IDE or SATA hard drives over a USB 2.0 connection. It works with most IDE (up to ATA 133) and SATA 1.0a devices.  This device provides a simple, yet powerful data transfer and backup solution.

TU2-IDSA (Version V1.0R)

Features

- Compliant with USB 2.0 and SATA (Serial ATA) 1.0a Specifications

- Compliant with USB Mass Storage Class Bulk-Only Transport Specification

- Supports combo connectors IDE (40pin & 44pin) for 2.5” / 3.5” /5.5” IDE & SATA devices

- Turns an internal IDE/SATA hard drive or CD/DVD-ROM into an external USB device

- Operates IDE and SATA devices simultaneously

- Compliant with Windows 98SE/ME/2000/XP/2003 Server Mac OS X

- Plug & Play for easy Installation

- 2-Year Warranty

Specifications					    Converter Solution for TU2-IDSA

| Hardware |  |
| --- | --- |
| Interface | USB 2.0, 1.1 Serial ATA 1.0a (Serial ATA) Ultra ATA 133 (Parallel ATA) |
| Supported OS | Windows 98SE/ME/2000/XP/2003 Server, Mac OS X |
| Connectors | SATA: 7-pin Signal Male IDE: IDE 44-pin Female IDE 2.5” to 3.5” Converter: IDE 44-pin Male to 40-pin Female USB: USB 2.0 Type A Male DC: Mini 4-pin Power Connector DC In: 2.1mm Power Plug |
| Data Rate | 1.5/12/480Mbps (USB) |
| Power Source | Input: AC 100~240V Output: 12V DC, 2A |
| Weight | Adapter: 70g(2.5oz.) Power Supply: 240g (8.5oz.) |
| Operating Temperature | 0 C~ 40 C (32 F~ 104 F) |
| Humidity | 85% (Non-Condensing) |
| Certification | FCC, CE |

Package Contents					System Requirement

- TU2-IDSA						- Computer with an USB 1.1 or 2.0 Port

- Multi-Language Quick Installation Guide		- CPU: 300 MHz or above
- Serial ATA Cable					- Memory: 128MB or above

- Hard Drive Power Cable				- Winodws 98SE/ME/2000/XP/2003 Server/Mac OS X

- 2.5” to 3.5” IDE Converter				- IDE or SATA 1.0a device

- AC/DC Power Supply